Atul Anurag

PES2UG19CS075

COMPUTER NETWORKS LAB

Week 10

IPv6 Configuration and Static Routing

Learning Objectives:

- Perform basic IPv6 configurations on a Desktop and Router.

- Distinguish between IPv4 and IPv6 addresses - Configure IPv6 static routes in Router - Observe traffic flow using IPv6 static routes.

- IPv6 neighbour cache entries

- Understanding IPv6 Link Local Address

- Working with ping6 and tracepath6

LAB Network Topology:

Steps :

- Create and deploy the given topology.

- Configure the PC/Workstation IP address as mentioned in topology.

Alice

IPv6 address – 2001::02/64 , Gateway – 2001::01

# Bob

IPv6 address – 2003::02/64 , Gateway – 2003::01

Example :

3. Enable IPv6 in Router-1

operational> configure

Entering configuration mode with exclusive access. configure> modify parameter-group router data Info: Parameter group instance loaded for modification.

configure> set ipv6 enable yes configure> save

Info: Parameter group router "data" saved

configure>

Check IPv6 information in router details

# operational> show router details data

4. Configure IPv6 interfaces in Router-1

* Configure IPv6 global address 2001::01/64 to interface if-port-1

operational> configure

Entering configuration mode with exclusive access.

configure> modify parameter-group interface if-port-1

Info: Parameter group instance loaded for modification.

configure> default ip ipv4 configure> enter ip ipv6

[ interface:"if-port-1" > ip > ipv6 ] configure> show draft -e

[ interface:"if-port-1" > ip > ipv6 ]  enable no

address 0000:0000:0000:0000:0000:0000:0000:0000  netmask 0000:0000:0000:0000:0000:0000:0000:0000  peer-address 0000:0000:0000:0000:0000:0000:0000:0000  peer-netmask 0000:0000:0000:0000:0000:0000:0000:0000  link-local-address 0000:0000:0000:0000:0000:0000:0000:0000  link-local-netmask 0000:0000:0000:0000:0000:0000:0000:0000  preference 1  metric 1  ndp {

cache-timeout 1200      unsolicited-learning enable

}  vrrp {      enable no

virtual-router [+] {

}

}

configure> set enable yes configure> set address 2001::01/64

configure> save

Info: Parameter group interface "if-port-1" saved configure>

## * Configure IPv6 global address 2002::01/64 to interface if-port-2

configure> modify parameter-group interface if-port-2

Info: Parameter group instance loaded for modification.

configure> default ip ipv4 configure> set ip ipv6 enable yes configure> set ip ipv6 address 2002::01/64 configure> save

Info: Parameter group interface "if-port-2" saved configure> exit

## * Verify Interface configurations

operational> show interface all

Interface name                   Status    Encaps-   IP address                                             ulation

------------------------------------------------------------------------------  if-port-1                        up        ethernet  2001::1/64

fe80::226:f7ff:fe00:6d/64  if-port-2                        up        ethernet  2002::1/64

fe80::226:f7ff:fe00:6e/64  if-port-3                        down      ethernet  -  if-port-4                        down      ethernet  -  if-port-5                        down      ethernet  -  if-port-6                        down      ethernet  -  if-port-7                        down      ethernet  -  if-port-8                        down      ethernet  -

management                       disabled  ethernet  10.0.0.12/24

Total number of interfaces displayed : 9

operational>

Check IPv6 information in “show interface details” command output

operational> show interface details if-port-1 if-port-2

5. Configure IPv6 static routes in Router-1

## * Configure a static route to reach  2003:00/64 network (Bob) with gateway as 2002::02( Router-2)

operational> configure

Entering configuration mode with exclusive access.

configure> create parameter-group ip-route v6-route-2003-nw

Info: Parameter group instance created. configure> show draft -e [ ip-route:"v6-route-2003-nw" ] *name "v6-route-2003-nw"  enable no  router ""  destination 0.0.0.0  netmask 0.0.0.0  next-hop {      router ""      gateway 0.0.0.0      label-switched-path ""

}

preference 30  metric 2  configure> set enable yes configure> set router data configure> set destination 2003::/64 configure> set next-hop gateway 2002::02 configure> save

Info: Parameter group ip-route "v6-route-2003-nw" saved configure> configure>

6. Display IPv6 routing table in Router-1

The configured static route should appear in the IPv6 routing table

operational> show route summary -F ipv6 data

> IPv6 active routes

>> Destination : ::1/128

Gateway(s)  : { ^loopback-16387

::1 }

Source      : direct

Flags       : -

>> Destination : 2001::/64

Gateway(s)  : { if-port-1

:: }

Source      : direct

Flags       : -

>> Destination : 2002::/64

Gateway(s)  : { if-port-2

:: }

Source      : direct

Flags       : -

>> Destination : 2003::/64

Gateway(s)  : { if-port-2

2002::2 }

Source      : static

Flags       : -

>> Destination : fe80::/64

Gateway(s)  : { if-port-1

:: }

Source      : direct

Flags       : -

>> Destination : fe80::/64

Gateway(s)  : { if-port-2

:: }

Source      : direct

Flags       : -

Total number of IPv6 active routes displayed : 6  No IPv6 backup routes are available operational>

7. Enable IPv6 in Router-2

operational> configure

Entering configuration mode with exclusive access. configure> modify parameter-group router data Info: Parameter group instance loaded for modification.

configure> set ipv6 enable yes configure> save

Info: Parameter group router "data" saved configure>

Check IPv6 information in router details

operational> show router details data

8. Configure IPv6 interfaces in Router-2

## * Configure IPv6 global address 2003::01/64 to interface if-port-1

configure> modify parameter-group interface if-port-1 Info: Parameter group instance loaded for modification.

configure> default ip ipv4 configure> set ip ipv6 enable yes configure> set ip ipv6 address 2003::01/64 configure> save

Info: Parameter group interface "if-port-1" saved configure> exit

## * Configure IPv6 global address 2002::02/64 to interface if-port-2

configure> modify parameter-group interface if-port-2

Info: Parameter group instance loaded for modification. configure> default ip ipv4 configure> set ip ipv6 enable yes configure> set ip ipv6 address 2002::02/64 configure> save

Info: Parameter group interface "if-port-2" saved

## * Verify Interface configurations

operational> show interface all

Interface name                   Status    Encaps-   IP address                                             ulation

------------------------------------------------------------------------------  if-port-1                        up        ethernet  2003::1/64

fe80::226:f7ff:fe00:76/64  if-port-2                        up        ethernet  2002::2/64

fe80::226:f7ff:fe00:77/64  if-port-3                        down      ethernet  -  if-port-4                        down      ethernet  -  if-port-5                        down      ethernet  -  if-port-6                        down      ethernet  -  if-port-7                        down      ethernet  -  if-port-8                        down      ethernet  -

management                       disabled  ethernet  10.0.0.12/24

Total number of interfaces displayed : 9 operational>

Check IPv6 information in “show interface details” command output

operational> show interface details if-port-1 if-port-2

9. Configure IPv6 static route in Router-2

## * Configure a static route to reach  2001:00/64 network (Alice) with gateway as 2002::01( Router-1)

operational> configure

Entering configuration mode with exclusive access.

configure> create parameter-group ip-route v6-route-2001-nw

Info: Parameter group instance created. configure> show draft -e

[ ip-route:"v6-route-2001-nw" ] *name "v6-route-2001-nw"  enable no  router ""  destination 0.0.0.0  netmask 0.0.0.0  next-hop {      router ""      gateway 0.0.0.0      label-switched-path ""

}  preference 30  metric 2

configure> set enable yes configure> set router data configure> set destination 2001::/64 configure> set next-hop gateway 2002::01

configure> save

Info: Parameter group ip-route "v6-route-2001-nw" saved configure> show draft -e [ ip-route:"v6-route-2001-nw" ] *name "v6-route-2001-nw"  enable yes  router "data"

destination 2001:0000:0000:0000:0000:0000:0000:0000  netmask ffff:ffff:ffff:ffff:0000:0000:0000:0000  next-hop {      router ""

gateway 2002:0000:0000:0000:0000:0000:0000:0001      label-switched-path ""

}  preference 30  metric 2

configure>

10. Display IPv6 routing table in Router-2

operational> show route summary -F ipv6 data

> IPv6 active routes

>> Destination : ::1/128

Gateway(s)  : { ^loopback-16387

::1 }

Source      : direct

Flags       : -

>> Destination : 2001::/64

Gateway(s)  : { if-port-2

2002::1 }

Source      : static

Flags       : -

>> Destination : 2002::/64

Gateway(s)  : { if-port-2

:: }

Source      : direct

Flags       : -

>> Destination : 2003::/64

Gateway(s)  : { if-port-1

:: }

Source      : direct

Flags       : -

>> Destination : fe80::/64

Gateway(s)  : { if-port-1

:: }

Source      : direct

Flags       : -

>> Destination : fe80::/64

Gateway(s)  : { if-port-2

:: }

Source      : direct

Flags       : -

Total number of IPv6 active routes displayed : 6  No IPv6 backup routes are available operational>

11. Verify traffic flow between Alice and Bob

- From Alice workstation ping Bob, observe the packet from and TTL in ping reply

- From Alice workstation run tracepath to Bob’s IP. Observer the intermediate hops

- Check IPv6 NDP table on Router-1

This is similar to ARP Table in IPv4.

operational> show ipv6 neighbour summary data

Host address                            MAC address       Interface

-------------------------------------------------------------------------------

2002::1                                 00:26:f7:00:00:6e if-port-2  2003::2                                 00:26:f7:00:09:3c if-port-1  fe80::226:f7ff:fe00:6e                  00:26:f7:00:00:6e if-port-2  fe80::5d97:cf2f:4a3:d8cb                00:26:f7:00:09:3c if-port-1

Total number of NDP entries displayed : 4

operational>

- Verify auto-configured  Link Local Address on IPv6 interfaces

All IPv6 enabled interfaces will have a link-local address. IPv6 link-local address is a unicast address that is configured automatically using the prefix FE80::/10 and port MAC in the modified EUI-64 format. The linklocal address can also be manually configured.

Link-local addresses are used for a addressing on a single physical link. These addresses can be used to reach the neighboring nodes attached to the same link.  Routers will not forward packets using link-local addresses.

Two routers can have same link-local address and can still communicate over directly connected network. But, the global unicast address should be unique in a network as they are routable.

Login to Router-1 and check the auto-configured link local address.

For Example :

operational> show interface details if-port-1

> Interface : if-port-1

General Information

-------------------

ID                     : 21

Encapsulation          : ethernet

MTU                    : 1500

Base port type         : fast-ethernet

Base port location     : { shelf-1 { active-controller base-slot } port-1 }

State Information

-----------------

State                  : up

Last state transition  : 15:19:44, Monday, March 18, 2019 IST

Work flags             : -- -- ----- -----

Ethernet information

--------------------

VLAN tagging           : disabled

IP information

--------------

Router                 : data

IPv6 information

----------------

Address                : 2001::1

Netmask                : ffff:ffff:ffff:ffff::

Link local Address     : fe80::226:f7ff:fe00:6d <====== Combination of FE08 and port MAC

Link local Netmask     : ffff:ffff:ffff:ffff::

Scope Zone             : 33488917

Preference             : 1

Metric                 : 1

TE information

--------------

Maximum Bandwidth                   : 10000 kbps

Maximum Reservable Bandwidth        : 10000 kbps

Update threshold percentage         : 10

operational>

operational> show fast-ethernet details { shelf-1 { active-controller base-slot } port-1 }

> Port : { shelf-1 { active-controller base-slot } port-1 }  Port details

------------

Name                :

MAC address         : 00:26:f7:00:00:6d  <=============

POST                : passed

Media               : copper

Loop back mode      : no-loopback

State               : up

Duplex mode         : half-duplex

Speed               : ten-mbps  Work flags          : ---- ---- operational>

14. Check the connectivity between Router-1 and Router-2 using   Link Local Address

Login to Router-2 and get the link-local address of interface connected to Router-1.

Now, Login to Router-1 and ping the link-local address on Router-2 and observe the response. When pinging  link-local address, the the name if out-going interface should be specified in the command. If no interface or wrong interface name is specified, ping will result in error or unsuccessful.